CS 350 Final Project

Smart Thermostat Architecture Recommendation

Prepared for SysTec Engineering

Introduction

SysTec's thermostat prototype, built on a Raspberry Pi 4B, demonstrates the required functionality: reading temperature over I2C, driving LEDs and buttons over GPIO, an LCD status display, and UART reporting. The production version does not need a full Linux single-board computer, it needs a small, low-power architecture with integrated Wi-Fi that supports the same peripherals and connects directly to SysTec's cloud servers. This report compares Raspberry Pi, Microchip, and Freescale (now part of NXP) production options against three requirements: peripheral support, Wi-Fi/cloud connectivity, and sufficient Flash/RAM.

Peripheral Support

All three architectures expose the I2C, GPIO, and UART interfaces this firmware depends on. The Raspberry Pi Pico W's RP2040 provides two I2C controllers, 26 GPIO pins, and two UART peripherals (Raspberry Pi Foundation, n.d.). Microchip's SAM W25 module pairs a SAM D21 Cortex-M0+ core with the same I2C/UART/GPIO set on a single chip (Microchip Technology, n.d.). Freescale/NXP Kinetis K-series MCUs, such as the K64F, expose the same interfaces and were the reference architecture behind widely used embedded prototyping boards (NXP Semiconductors, n.d.). Peripheral support is therefore not a differentiator, all three can drive this project's sensor, LEDs, buttons, and LCD.

Cloud Connectivity via Wi-Fi

Connectivity is where the architectures diverge. The Pico W integrates an Infineon CYW43439 Wi-Fi radio on-board (Raspberry Pi Foundation, n.d.). The SAM W25 goes further, combining the MCU and a pre-certified Wi-Fi radio (ATWINC1500-based) into one module, simplifying both hardware design and FCC certification (Microchip Technology, n.d.). Kinetis K-series MCUs have no integrated Wi-Fi; reaching the cloud requires a separate companion chip such as the NXP 88W8801, adding board space, a second bill-of-materials line, and bridging firmware (NXP Semiconductors, n.d.). For a requirement centered on cloud connectivity, integrated Wi-Fi removes real hardware and certification risk.

Flash, RAM, and Overall Fit

This firmware, a small state machine, an I2C driver, GPIO/LED control, LCD output, and a UART loop, is modest, so none of the three would be flash- or RAM-constrained. The RP2040 offers 264 KB SRAM, typically paired with 2 MB external flash (Raspberry Pi Foundation, n.d.). The SAM W25 provides up to 256 KB flash and 32 KB SRAM (Microchip Technology, n.d.). Kinetis K64-class devices offer up to 1 MB flash and 256 KB SRAM (NXP Semiconductors, n.d.), the most headroom of the three, though more than this firmware needs.

Recommendation

This report recommends the Microchip SAM W25. It meets all three requirements: it exposes the needed I2C/GPIO/UART peripherals, its Wi-Fi radio is pre-certified on the same module (shortening hardware design and regulatory approval), and its Flash/RAM comfortably fit this firmware. The Kinetis K64 offers more raw capacity, but that headroom is unused here and comes at the cost of a separate Wi-Fi chip, a weaker fit for a cloud-connectivity requirement. The Pico W is a close second, also integrating Wi-Fi at low cost, and is a reasonable alternative if SysTec later needs the RP2040's extra processing headroom. Between the two, the SAM W25's single, pre-certified module gives it the edge for reaching production fastest.

References

Microchip Technology. (n.d.). ATSAMW25 Xplained Pro Wi-Fi module [Product page]. Microchip Technology Inc. https://www.microchip.com/

NXP Semiconductors. (n.d.). Kinetis K64 sub-family: ARM Cortex-M4 MCU [Product page]. NXP Semiconductors. https://www.nxp.com/

Raspberry Pi Foundation. (n.d.). Raspberry Pi Pico W datasheet. Raspberry Pi Ltd. https://www.raspberrypi.com